Accelerating the Value Iteration Algorithm on the Stochastic Economic Lot Scheduling Problem for Continuous Multi-Grade Production

# Georgios Kalantzis

# Copyright © June 2012

Title	Accelerating the Value Iteration Algorithm on the Stochastic Economic Lot Scheduling Problem for Continuous Multi-Grade Production

# Author 			Georgios Evripidis Kalantzis

# Student Number		343453

Supervisor		Dr. Adriana Gabor, Erasmus University Rotterdam

Co-reader		M.Sc. Judith Mulder, Erasmus University Rotterdam

Study			Econometrics and Management Science

Specialization 		Master in Operational Research and Quantitative Logistics

# University		Erasmus University Rotterdam

Contents

- Introduction4

- Problem definition6

  - Process VS discrete manufacturing6

  - The Stochastic Economic Lot Scheduling Problem6

  - SELSP for continuous multi-grade production9

- Literature  review11

- Methodology13

  - Markov Decision Processes13

  - Summary of algorithms for decision problems 15

  - Standard Value Iteration Algorithm17

  - Accelerated Value Iteration Algorithms23

    - Modified Value Iteration Algorithm23

    - Minimum Ratio Criterion Value Iteration Algorithm24

    - Minimum Difference Criterion Value Iteration Algorithm26

    - K-step Minimum Difference Criterion Value Iteration Algorithm31

- Mathematical Model for SELSP33

- Heuristics37

  - Action Elimination37

  - 2-Grade Action Elimination Heuristic37

- Numerical Experiments 43

  - Data description43

  - Influence of the initial state on SELSP45

  - Algorithm Performance Comparisons46

    - 2-Grade SELSP46

    - 3 and 4-Grade SELSP51

- Conclusions and Future Research54

Bibliography55

APPENDIX 1: Tables with detailed results. 57

1. Introduction

In this master thesis production scheduling is researched and specifically a variant of the Stochastic Economic Lot Scheduling Problem (SELSP) is addressed (Liberopoulos et al.) (2009) [11]. The SELSP is used to model a single machine with restricted productiveness, used to produce different products under random stationary demands. The products are stored in a warehouse with limited storage capacity. It is assumed that spillover, lost sales and switchover costs and times occur. SELSP together with the Stochastic Capacitated Lot Sizing Problem (SCLSP) constitute the two variants of the Stochastic Lot Scheduling Problem (SLSP) (Sox et al.) (1999) [12]. While SELSP is more suitable to model the continuous multi-product production of process industries where the different grades of a product are produced ceaselessly, SCLSP is suitable for the rest of the industries where the production takes place in a clearly discrete manner. The SELSP variant under consideration in this thesis, is modeled as a Markov Decision Process (MDP). Hatzikonstantinou (2009) [3] finds optimal policies for the SELSP via the Standard Value Iteration Algorithm (SVIA). The outcome is satisfying in terms of the optimal policy’s quality, but not encouraging in terms of the computational time needed to find such a policy, especially when the state space grows bigger. This master thesis focuses in algorithms, heuristic procedures and techniques that efficiently find optimal and ε-optimal policies, improving on the same time the SVIA’s number of iterations and the needed computational CPU time. The algorithms that are adopted to reduce the computational effort are the Minimum Difference Criterion Value Iteration Algorithm (MDCVIA) which uses a Dynamic Relaxation Factor (DRF) to accelerate the procedure and the  K-step MDCVIA which enhances with K-value oriented steps per iteration the MDCVIA. A heuristic procedure is developed which performs Graphical Action Elimination (GAE), based on the obtained policy. The aforementioned MDCVIA and its version enhanced with GAE are compared against SVIA on realistic experiments, to conclude that they confront more effectively - when compared to SVIA - the well known curse of dimensionality.

In Chapter 2 the difference between process and discrete manufacturing is described along with the definition of the SELSP for continuous multi-grade production. Chapter 3 contains a literature review on the SELSP. In Chapter 4, after the presenting MDPs and SVIA, the effort focuses on the algorithmic theory used to enhance the SVIA’s effectiveness on solving large-scale MDPs. Chapter 5 describes the SELSP formulation as a discrete time MDP. Chapter 6 follows with the description of a heuristic based on Action Elimination (AE). In Chapter 7 numerical experiments, comparisons and results are presented and conclusions are drawn. Finally, in Chapter 8 a short discussion on directions for further research is included.

2. Problem definition

The manufacturing environment may differ from one industry to another in several steps and functions within the production procedure: from the way the raw material is delivered (trucks, trains, vessels or pipelines) or the way it undergoes processing in the production facility (continuously or discretely), till the way the finished goods are stored via small scale (packages, bottles, cans) or large scale (warehouses or silos) storage methods.

2.1. Process versus discrete manufacturing

In industrial terms, industries are separated between process and discrete industries. Process manufacturing environment refers to the industries that produce food and beverages, paints and coatings, special chemicals, textiles, cosmeceuticals, nutraceuticals, pharmaceuticals, textiles, cement, mineral products, coal products, metallurgical products, petrochemicals etc, where the raw material flows continuously and the production is in bulks. Discrete manufacturing environments are found in industries that produce industrial and consumer electronics, household items, cars, airplanes, equipment and accessories, toys, computers, assemblies etc characterized by high or low complexity.

Regarding the production process itself, other differences can also be addressed. In process industries once the resulting product is made, it cannot be distilled or decomposed back to its basic components, because they are not distinguishable anymore (paint ingredients cannot be separated once the paint is produced). In discrete industries on the other hand, the final product can be disassembled back to its modules or components. This difference is due to the way the raw material is treated in each industry. In process industries the raw material flows continuously in the production line, while in discrete industries modules and parts enter the production line after being selected from finished goods inventories. Thus in the first case one must know the formula and the proportion of needed ingredients, but in the second the bill of materials is needed to compose the final product. This basic difference is also applied in multi-product production environments to distinguish between continuous and discrete processes, because usually a single machine produces multiple products.

To give an example: if half a ton of white and another half of black paint are ordered and there is no availability of the black coloring that is added to the white paint during the production process; half a ton of white paint can be produced satisfying the demand partly. Moreover if half of the black coloring needed to produce half a ton of black paint is available, the industry is able to produce all the white paint ordered and half of the black one, again satisfying a part of the total demand. On the other hand if a white and a black bicycle are ordered to a bicycle manufacturer, the product cannot be completed if there are no wheels available. This results in lost demand for the two products. To further distinguish the two production environments, the continuous industries products are measured in mass or volume units, while the discrete industries products are measured in units of a product.

2.2. The Stochastic Economic Lot Scheduling Problem

There exist numerous variations of single-machine multi-product scheduling problems. A universal categorization of these problems depends on three main attributes of the production environment (Winands et al.) (2011) [17]. The first attribute is the occurrence or not of setup costs and times, when the production on a single machine changes from one product to another. If setup times and costs occur, the production is interrupted for an amount of time resulting in reduced production capacity. The second attribute is the kind of products that are produced. Standardized products allow scheduling the batch production of the machine, while customized products according to the customer’s specifications are subject to changes and require low volume production. Stochastic or deterministic environment is the last attribute under consideration. In the case of deterministic environments the scheduling of the machine requires a solid production schedule that will be applied repeatedly. A stochastic environment however, demands a production schedule that will dynamically respond to the stochastic changes of demand, setup times and possibly other factors. Thus, by combining these attributes eight single-machine multi-product scheduling problem categories occur. The most common production environment is described by a single-machine with considerable setup times and costs that produces standardized products, in an environment that is characterized totally or partially by stochasticity.

When a single machine with limited production capacity is able to produce multiple products that are stored in a warehouse with limited capacity and remarkable times and costs occur during a switchover of the machine to produce another product, the need of scheduling the production of the machine arises. The definition of this single-machine multi-product lot scheduling problem (SLP) under deterministic demand for each product, differentiates according to the assumption adopted in each production environment regarding time. Thus the Economic Lot Scheduling Problem (ELSP) is used when the time is considered continuous and the Capacitated Lot Sizing Problem (CLSP) is used when time is discrete.  As a result, ELSP and CLSP are used to describe process and discrete production environments respectively.

Unfortunately the deterministic demand assumption for every product is proved unreliable, because of the demand uncertainty in a real life problem. Under the deterministic demand assumption, the problem needs to be solved again in order to include the demand changes. The issue of demand stochasticity should be considered, in order to formulate a problem where the changes in demand are effectively incorporated. Similarly to the SLP, the Stochastic Lot Scheduling Problem (SLSP) is again divided into two categories, according to the time assumption that is adopted. The resulting problems are the Stochastic Economic Lot Scheduling Problem (SELSP) and the Stochastic Capacitated Lot Sizing Problem (SCLSP), which emerged from their deterministic versions. In the SELSP, an infinite planning horizon under stationary demand is assumed, but in the SCLSP the planning horizon is assumed finite, under non-stationary and independent demand.

The SELSP can be further categorized into sub-problems according to the sequence and to the lot sizing policy that are followed, in order to schedule the production. The production sequence in which a machine produces multiple products can be fixed or dynamic. A cyclical sequence imposed on the machine to produce in a predefined way the individual products is called fixed. Thus in a SELSP case regarding three products a fixed sequence is B-C-A-C-A, for respective predefined product quantities in every cycle. Furthermore the cycle length can be dynamic or fixed. A dynamic cycle length allows different product quantities to be produced under the same sequence each time a cycle is repeated.  Dynamic sequence is the last category of sequences, where both the sequence and the cycle length are variable in every cycle. The other main production characteristic is the lot-sizing policy that is adopted and is divided into local and global lot sizing policies. A local lot sizing policy depends on the inventory level of the product that undergoes production. A global lot sizing policy depends on the entire state of the system, which is the product that is under production and the inventory levels of all products.  This survey focuses on the category of SELSPs regarding dynamic sequences and global lot sizing policies.

2.3. SELSP for Continuous Multi-Grade Production

Instead of the classical SELSP version for continuous multi-product production, we consider multiple grades. Grades of a product are in fact variations of a single product, produced continuously in a single machine. They are distinguishable from each other according to one or more of their main attributes (color, density, quality, chemical properties). This is common practice for a great number of process industries. In the majority of these, the machine produces sequentially the different grades (Liberopoulos et al.) (2010) [10]. Thus if the three grades of a product are A, B and C and the machine is set to produce grade A the only allowable switchover is to set the machine production to grade B. Grade C is unreachable directly from grade A and the other way around. If a switchover from grade A to C is required, the machine has always to traverse through the middle grade B. Since the production is continuous, an appreciable amount of time is needed in order to switch the production from one grade to another. When such a switchover takes place, an intermediate undesired grade is produced. In this thesis, it is assumed that the switchover times are deterministic and equal.

In order to facilitate the intermediate grades in a model formulation there are two approaches. The first is to divide the intermediate grade into two equal portions and assume that when the machine setup switches from grade A to grade B, the first half is considered as grade A and the second one as grade B. The second approach is to assume that when switching from A to B, the intermediate grade is grade A and when changing from B to A, the intermediate grade is considered to be grade B. One of the above assumptions has to be adopted in order to balance the amounts of grade A and grade B produced in an infinite horizon context. The costs of the SELSP for continuous multi-grade production are related to the switchovers of the machine, to the warehouse capacity and to the service level. A switchover cost occurs each time the production is set to a neighboring grade. Lost sales costs per unit shortage occur each time a demand is not realized. Finally, spill-over costs per unit of product being in excess are integrated in the cost formulation of the model.

In conclusion, different lot scheduling problems are formulated according to the single-machine multi-product production environment, in order to describe the way each production facility functions. The main problem categories are the SCLSP for finite planning horizon under non-stationary demands and the SELSP for infinite planning horizon under stationary demands. Moreover the characteristics of continuous multi-grade production can be considered together with the SELSP, resulting in a SELSP variant that describes common real-life production applications in process industries. Several approaches are presented in the following literature review Chapter, in order to formulate a model for SELSP and to find a schedule with minimal costs.

3. Literature Review

In the decision making literature there exist numerous surveys regarding the SELSP, that evolved through the decades from its deterministic forefather the Economic Lot Sizing Problem (ELSP). Over the years, researchers studied different aspects and characteristics of the SELSP, that vary from industry to industry, or they conducted case studies that allocated and modeled specific features of the continuous production process. Thus a wide range of models have been proposed for different production environments in order to model SELSP variants adequately.

Leachman and Gascon (1988) [8] approach SELSP adopting a global lot sizing policy to determine a fixed sequence with dynamic cycle lengths. The heuristic they develop under a periodic review control policy, combines dynamically solved deterministic ELSP solutions that assume non-stationary demand. The discrete time model they use determines the quantity of each product that should be produced in each time period, but the action to idle the production facility may also be a decision. In the case where the ELSP solutions are proved inadequate to prevent lost sales, these solutions are calculated again.

Sox and Muckstad (1997) [13] develop a finite horizon discrete time mathematical programming formulation for the SELSP. Moreover, they introduce the realistic assumption that a machine setup is needed at the beginning of each period, even if the same product keeps being produced. They introduce a relaxed version of the model, in order to ignore this assumption whenever it is needed.  They solve the model using a decomposition algorithm based on Lagrangian relaxations, in order to generate a dynamic production sequence under a global lot sizing policy.

Liberopoulos, Pandelis and Hatzikostantinou (2009) [11] introduce a SELSP variant for continuous multi-grade production similar to the one presented in Chapter 2.3.  The SELSP variant is modeled as a discrete time MDP and is categorized in the area of dynamic sequencing under a global lot sizing policy. The difference compared to the classical SELSP is that each time the machine can only change the production setup only to a neighboring grade. The model can be easily changed to simulate classical SELSP production environments where the single machine produces grades of a product or products, independently of grade’s neighboring criteria. However, in this survey a change is proposed regarding the usage of the successive approximations solution method the authors use. The cost of a state is no more compared and dependent to a given initial state, in order to comply with the general theory regarding the relative value functions of states of a MDP. This change does not influence the behavior of the MDP and the solution method, but now the model is in compliance with the corresponding literature.

In this literature review, SELSP models that consider global lot sizing policies are presented. The main modeling approaches of mathematical programming and MDP formulations for SELSP are discussed, along with their corresponding solution methods. Moreover the elementary heuristic procedure which combines ELSP solutions to generate SELSP solutions is mentioned. The successive approximations method is an algorithm to solve MDPs, also known as The Standard Value Iteration Algorithm (SVIA). In the next Chapter the main solution methods to solve an MDP are presented, putting emphasis on the SVIA and its variants.

4. Methodology

The Markov decision model is an efficient tool for modeling dynamic systems characterized by uncertainty. The decision model is a result of blending the underlying concepts of the Markov model and dynamic programming. MDPs have been applied in problems regarding maintenance, manufacturing, inventory control, robotics, automated control, medical treatment, telecommunications etc. Their wide applicability proves the usefulness of the model. The majority of surveys focus on discrete time MDPs, due to the high complexity of continuous time MDPs.

In Section 4.1. an introduction to MDPs and the optimal policy is given, while 4.2. contains a summary of algorithms to find that policy. Section 4.3. describes the basic functions of SVIA. Finally Section 4.4. provides a review of accelerated SVIA variants and criteria.

4.1. Markov Decision Processes

In general, a MDP behaves similarly to a Markov Process, but at every time epoch a stochastic decision has to be made. The objective is to find an optimal policy of sequential decisions that optimizes a specific performance criterion, for example the minimization of the expected average cost. A Markov Process simulates the outcome of a predefined stochastic model, allowing only the calculation of a single predefined policy. The drawback is that it is computationally impossible to simulate every feasible policy on a large-scale problem. MDPs perform stochastic optimization of the entire model that is guaranteed to result in an optimal policy and calculate the outcome of that policy. The drawback for MDPs is that the computational effort to solve MDPs increases, as the size of the problem increases.

MDPs are used to model dynamic systems that evolve over time under uncertainty, where at various time epochs a decision is made to optimize a given criterion. MDPs are stochastic control processes used to provide sequential decisions and are categorized according to the time assumptions adopted for the control policy. The system dynamics can be continuous or discrete, resulting in respective continuous or discrete time MDP model formulations and review control policies. In continuous time MDPs, the decision maker (agent) can choose an action at whichever point in time. In discrete time MDPs the decisions are taken in discrete equidistant review (decision) epochs. SMDPs also consider discrete time, but the time interval between two consecutive reviews is random. Finally the planning time horizon can be considered finite or infinite. The infinite horizon assumption is adopted, when the time horizon is not known or it is very big. An infinite horizon though requires infinite number of data, thus the data are assumed time homogeneous. In most of the cases discrete time MDPs are used under an infinite horizon assumption. As a result the majority of solution methods are able to solve only this category of MDPs.

In order to define the discrete time MDP under infinite planning horizon, the following system is considered. At each review epoch the system belongs to a state i and the decision maker chooses one of the available decisions (actions) a that belong to a state i. The set of possible states is denoted I and the set of possible actions a for a state i ϵ I, is denoted Ai.  Both I and Ai, ∀ i ϵ I are assumed finite. In state i a reward (cost) Ci(a) is earned (incurred) and the system jumps to a state j with probability pij(a), where jpij(a)=1. Moreover, state j depends on the action a chosen by the agent and the state current state i. The one step reward and the one step probabilities are characterized by homogeneity over time. By assuming that the next state j the system will visit depends only at the current state i of the system, MDPs satisfy the Markov assumption. Moreover the states of a MDP should be carefully modeled in an infinite horizon context, in order to end up with stationary state transitions. The resulting stationary policy Ri determines a specific action a for every state i, and uses it every time the system is in state i. When a stochastic process is combined with an optimal policy Ri the result is a Markov Chain (MC), with one step transition probabilities pij(a), that earns (incurs) a reward (cost) Ci(a) every time the system visits state i.

In order to find an optimal policy for the SELSP, there exist several solution methods that in general are able to solve discrete-time MDPs in optimization problems, by providing an optimal policy Ri applied to all the states i, i∈I of the system. Although a lot of algorithms are able to provide an optimal or near-optimal policy, it is of great importance to acquire this policy Ri, with as less computational effort as possible. The two classical approaches to find such an optimal policy for an MDP are dynamic and linear programming. Several algorithmic procedures to find an optimal policy have been developed, through over half a century of research in decision-making.

4.2. Summary of algorithms for decision problems

The Standard Value Iteration Algorithm (SVIA) is a recursive algorithm, based on the famous Bellman equation that Richard Bellman introduced in the decade of 50’s. His work stimulated the research in the area of MDPs resulting in numerous variants and modifications of SVIA. It is also known as backwards induction - where a process of reasoning backwards in time is used until the convergence of the algorithm is achieved, in order to determine a sequence of optimal actions.  SVIA is one of the main methods to find an approximate optimal policy for an MDP, with remarkable performance in systems with large state sets I.

Policy Iteration Algorithm (PIA) (Tijms (2003) [16]) introduced by Howard in the 60’s and refined by Puterman in the 70’s is based on choosing an initial policy Ri and continuously constructing new improved policies iteratively, until optimality is achieved. It encloses both the aspects of linear and dynamic programming and is famous due to its robustness. PIA solves in each iteration k, a system of linear equations equal to the size of the state space I of the MDP. When PIA comes to the case of solving large-scale MDPs, the algorithm solves large systems of linear equations, which is the main drawback of the algorithm. Like SVIA a lot of variants and modifications exist for the PIA.

Another method to find an optimal policy is prioritized swapping, where one performs SVIA or PIA focusing on states of great importance, based on the value functions that the algorithm computes for every state i∈I or on the usage frequency fi of these states or on states of interest to the person using the algorithm. By concentrating the effort on a subset of states I where,  I⊆I rather than the entire state space I, to find those important Vkis, i∈ I, considerable computational effort is saved. The importance of a state i can be determined by various criteria developed according to the problems’ features (e.g. total reward criterion).

Linear programming (LP) (Tijms (2003) [16]) is another approach to find an optimal policy for an MDP. It is also possible to find a non-stationary optimal policy Ri, i∈I, if probabilistic constraints together with Lagrange multipliers are used. It is obvious that like in the PIA case as the state space I of a system grows bigger the number of corresponding linear equations grows, resulting in a chaotic system of equations and constraints.

Reinforcement learning, which is suitable for long-term decision planning, uses exploration methods. In this environment the most profitable action is chosen with probability 1-p, while the rest of actions are chosen in total with probability p. The probability p may vary as the steps of the algorithm grow under a fixed schedule or it is adapted according to a heuristic procedure similarly to the mechanism of the simulated annealing algorithm.  Pattern search can be integrated with dynamic programming and convex optimization to formulate algorithms that search the multi-dimensional finite state space (Arruda et al. (2011) [1]). In every iteration k, variable sample sets of states are produced that provide descent search directions.

When considering practical real-environment problems, most MDPs are characterized by huge sparse transition matrices. Algorithms have been developed to perform a more mathematical - to the core of the MDP - approach and to take advantage of the structure of the (one-step) transition probability matrix produced by an MDP. After applying the basic concepts of periodicity, irreducibility, state classification and identifying the communicating and transient classes of the MDP - based on the elegant analysis proposed by Leizarowitz (2003) [9] - the states i, i∈I can be re-ordered in such a way that the transition matrix will become dense in the corresponding points(states) that belong in classes I of states, where I⊆I. Such a reordering of states makes possible the decomposition of the large scale MDP into smaller MDPs. After solving each perfectly structured sub-problem by SVIA, the separate policies Ri, i∈I can be connected through a heuristic procedure, like the one developed by Tetsuichiro, Masayuki and Masami (2007) [14].

Additionally to these algorithms, a number of techniques exist in order to enhance their convergence rate. The techniques are a test procedure performed in the end of an algorithm’s iteration. Action Elimination (AE) is used to track down the actions a of a MDP proved to give non optimal policies. As a result they are not taken into account in future iterations of an algorithm, reducing in this way the computational effort and increasing an algorithm’s efficiency. Another method is investigating the initial values that are set to initialize an algorithm. By setting the right initial values V0i, i∈I in the initialization step an algorithm is provided with a good kick-off, forcing it to converge faster within less iterations.

In the following Sections an effort to present SVIA is attempted, by providing detailed analysis of basic elements, attributes and functions.

4.3. Standard Value Iteration Algorithm

This Section contains a discussion on basic assumptions and characteristics in SVIA, such as initial values, bounds, stopping criteria, recursive schemes and “ties”.

When solving a MDP via SVIA, the ε-optimal policy is acquired under the reasoning of backward induction. The recursive equation that the Standard Value Iteration Algorithm uses to approximate the minimal average cost Gk* for k=1,2,… iteration, is:

Vki=mina∈AiCia+j∈Ipija∙Vk-1j, i∈I 				1

Bellman (1957) [2] and Tijms (2003) [16], denoted by Vki the minimal total expected costs when k time epochs remain, starting from the current state i and ending at state j, incurring a terminal cost V0j.

The key to the efficiency of SVIA is that it uses a recursion scheme to compute a sequence of value functions Vki, Vk+1i, …, i∈I, that approach the minimal average cost per time unit denoted by Gk. This is accomplished by computing lower bounds mk and upper bounds Mk in each iteration k, based on the differences δk(i) of two consecutive value functions Vki and Vk-1i, i∈I.

δk(i)=Vki-Vk-1i, i∈I 							(2)

mk=δk(l)=min i∈I{Vki-Vk-1i}, i∈I, state l (low) corresponding to the minimal difference									3 Mk=δk(h)=max i∈I{Vki-Vk-1i}, i∈I, state h (high) corresponding to the maximal difference									4

In order to force the bounds to approximate the minimal average cost Gk* and to find the desired accuracy ε-optimal policy, the tolerance error ε in which Gk* ranges is fixed and the stopping criterion becomes:

0≤(Mk-mk)≤ε    							          5

which is the supremum norm or relative tolerance criterion and ensures that

Vki-Vk-1i≤ε,						                      (6)

is satisfied ∀ i∈I. This criterion is rather strict. Therefore, a more relaxed stopping criterion that also satisfies (6) is used, described by (7) and is known as semi-span norm or the relative difference criterion.

0≤(Mk-mk)≤ε∙mk    							          7

When equation (7) serves as stopping criterion rather than equation (5), SVIA converges faster, managing on the same time to satisfy adequately equation (6). This explains the wide use of (7) amongst researchers. The number of iterations k that the algorithm needs till an optimal policy Ri is calculated, is problem dependent and grows as the state space I of the MDP grows. Moreover, k grows as the value of ε is reduced. Finally, when the number of one step transitions from a state i increases, the computational time needed to find an optimal policy increases as well.

As a result of the convergence of the algorithm, the corresponding actions a, a∈Ai that minimize the right hand side of (1) ∀ i∈I, will comprise the stationary optimal policy Ri. These policies are also named ε-optimal, because the cost found is close enough to the optimal Gk*. Moreover if the MDP is characterized by aperiodicity the convergence of SVIA is guaranteed, as mk and Mk converge geometrically always satisfying mk+1≥mk and Mk+1≤Mk. Consequently the same geometrical convergence applies to the optimal cost Gk*, as it constitutes a synthesis of two geometrical monotonic functions and is easily calculated from the relation:

Gk*≅(Mk+mk)/2 								          (8)

Tijms (2003) [16] adopts the Weak Unichain Assumption (WUA) when solving MDPs, to support theoretically the solutions found using Linear Programming and SVIA. WUA assumes that “For each average cost optimal stationary policy the associated Markov Chain has no two disjoint closed sets”. Thus, SVIA is able to calculate minimal expected average costs and optimal policies that are independent from an initial or special state. In case of not adopting WUA, for inventory problems under stationary bounded demands, the outcome is the generation of stationary policies where the inventory levels are dependent to the initial level (initial state). WUA is a realistic assumption to adopt in a real-life application like our SELSP variant. To conclude, WUA allows the establishment of a solid model that will both guarantee from a mathematical point of view the finding of optimal policies and an acceptable value for the minimal infinite horizon expected average cost.

The initial values V0i, i∈I that are necessary for the algorithm’s initialization are chosen arbitrarily inside the range [0,mina∈Ai{Cia}], but usually they are set equal to 0. Herzberg and Yechiali (1996) [6] remark on the significance of this issue suggesting further investigation of this “Phase 0”, because when the right values are chosen, the algorithm enjoys a decent initialization resulting in better convergence rates. Unfortunately, the relevant literature proposed by the authors could not be found and only intuitive experiments were performed.

Following the above analysis, the steps of the SVIA can be summarized as:

# Step 0 (initialization). Fix

V0i, i∈I, to satisfy: 0≤V0i≤mina ∈ Ai{Cia} 			 and set  k=1.

# Step 1 (value improvement step). Compute

Vki=mina∈Ai{Cia+j∈Ipija∙Vk-1(j)}, i∈I 		            (1)

# Step 2 (apply bounds on the minimal costs).Compute

mk=min i∈I{Vki-Vk-1i}, i∈I 					(3)

Mk=max i∈I{Vki-Vk-1i}, i∈I					(4)

# Step 3 (stopping test). If

0≤Mk-mk≤ε∙mk 							(7) Stop.

Step 4 (continuation). Set n=n+1 and go to step 1.

An example case of a SELSP is introduced at this point, in order to present the way that SVIA works. The example case considers a 2-Grade SELSP for a warehouse with capacity of 40 units of products, under the following distribution of demands Dn for each grade n of a product:

| n Dn | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.1 | 0.15 | 0.15 | 0.2 | 0.15 | 0.15 | 0.1 |
| 2 | 0.15 | 0.15 | 0.4 | 0.15 | 0.15 | 0 | 0 |

Table 1: Probability distributions of Dn, for the two grades.

The switchover cost per setup change is 10, the spill-over cost per unit of excess product is 5 and the lost sales cost per unit of unsatisfied demand per grade n is 10. The production capacity of the machine is 5 units of grade per time period and the error tolerance is ε=10-3.  The example case is solved via SVIA within k=169 iterations and tCPU=6.7 sec, producing the diagrams below. The two bounds mk and Mk converge geometrically to the minimal infinite horizon expected average cost Gk*, where  Gk*=2.7074.

Figure 1: Geometrical (monotonic) convergence of δk(i)s (left) and Gk* (right).

In order to extend the insight on how SVIA functions, the issue of “ties” is investigated. Quite often, the same value of a lower or/and upper bound appears in an iteration k for more than one states h or l, resulting in a “tie” for δk(h) or δk(l). When studying minimization problems, the majority of “ties” occur when searching for the lower bound mk, while few “ties” occur for the upper bound Mk.  The opposite behavior is expected for maximization problems. The number of “ties” is high in the first k iterations of SVIA and descents quickly (not linearly) as k grows.

The example case is again used to demonstrate the behavior of “ties” when using SVIA. In this case, a “tie” between the values of δk(i)s regarding mk occurred, 960, 420 and 147 times for k=1,2,3 respectively. For the rest of the iterations k, k ∈[4,…, 169], the “ties” appeared are depicted in Fig.4. Regarding Mk, a single “tie” occurred in the first iteration. The algorithm produces equal values between several δkis per iteration k in a state space I, where I=1772 states i. The higher frequency of “ties” in the first iterations of the algorithm indicates the need to set suitable initial values in the “Phase 0” of SVIA. This action forces the values of δkis to differentiate from each other within less iterations resulting in less “ties”, opposed to the case where V0i=0, i∈I.

Figure 2: Number of “ties” per k occurred regarding mk, k ∈[4,…, 169].

Note that the ideas of SVIA can be successfully applied in the case of discounted MDPs, in which the expected costs at time n are discounted by a factor βn. More specifically the SVIA recursion scheme becomes:

Vki=mina∈Ai{Cia+β ∙ j∈Ipija∙Vk-1(j)}, i∈I 			          (9)

This value iteration scheme is known as Pre-Jacobi, becoming the only applicable scheme for undiscounted MDPs. Herzberg and Yechiali (1994) [5] and Jaber (2008) [7] discuss on other improved variants of this scheme for discounted MDPs, that are amenable to use within SVIA’s concept, namely Jacobi, Pre-Gauss-Seidel and Gauss-Seidel. SVIA and its numerous variants perform better, when they are used to solve discounted MDPs.

An undiscounted MDP is a special case of a discounted one, for β=1 in equation (9). Discounted MDPs are used to model reward maximization problems, opposed to undiscounted MDPs that are used to model cost minimization problems. The discount factor is used, based on the fact that an earned reward will eventually have a reduced value in the long run, forcing SVIA to a faster convergence compared to the case where β=1 or close to 1. The latter explains the difficulties of the undiscounted case and the reason why the effort should be focused on accelerating the solution procedure. This is essential, especially when small error tolerances ε are acceptable or when the MDP is characterized by large state spaces I.

4.4. Accelerated Value Iteration Algorithms

In this Section a discussion on the acceleration of SVIA is conducted, continuing the methodology analysis of the previous Section. The discussion regards modified versions of SVIA, the concept of relaxation, relaxation criteria, computational considerations, “ties” and the type of convergence of bounds.

4.4.1. Modified Value Iteration Algorithm

Tijms and Eikeboom (1983) [15] and Tijms (2003) [16] suggest in their work, the usage of a Fixed Relaxation Factor (FRF) or a Dynamic Relaxation Factor (DRF) notated by ω, in order to enhance the speed of SVIA. The acceleration of the algorithm is needed, because the computational effort SVIA requires is problem dependent and proportional to the state space I of the MDP and inversely proportional to the defined accuracy number ε. The relaxation factor ω, ω>0 is used to update the value functions Vki, at the end of each step k by setting:

Vki=Vk-1i+ω∙Vki-Vk-1i					          10

for every i to approximate faster the respective Vk+1i, which in its turn results in faster convergence between the bounds Mk+1 and mk+1. The convergence of the bounds is not similar to SVIA’s convergence and is no more characterized by monotonous bounds. Thus, the algorithm is not mathematically proved to converge, but non-convergence rarely happens if 1≤ω≤3. This modified version of SVIA can also work for a SMDP, after it is converted into a MDP via the appropriate data transformation. In SMDPs where the time between decisions is exponentially distributed, fictitious time epochs are considered. Fictitious epochs are inserted using the memoryless property, in order to accelerate the solution procedure even more.

Many attempts are needed in order to find the optimal value of an FRF for a specific state space I and accuracy number ε of a problem. A DRF is efficiently derived dynamically in each iteration k, based on Vki, Mk and mk regardless the given state space I and ε. Exploiting the dynamics of  Mk and mk, in an effort to predict the future values Vk+1i at iteration k+1, the DRF is set as:

ω=Mk- mkMk- mk + j∈IpljRl – phjRh ∙ Vkj – Vk-1j  		            	          11

where, Rl the optimal decision at state l, and Rh the optimal decision at state h. When in an iteration k a “tie” occurs between the candidate states for Mk or mk, it is not clear which is the right state h or l to choose for equation (11). The states with equal δk(i) values form a set of candidate states Candk(h) and Candk(l) for h and l respectively, which is further investigated via the following modification. One of the candidate states h or l in iteration k is chosen from Candk(h) or Candk(l), if it was also chosen for Mk or mk respectively, in the previous iteration k-1. Else, the first state h or l from Candk(h) or Candk(l) that its value equals Mk or mk respectively is chosen.

When modified VIA calculates the optimal ω without the aforementioned modification regarding “ties”, it fails to choose the right state h or l. The algorithm after sweeping all states i in every k, wrongly selects the last state h or l that satisfy equation 3 or (4) respectively. As a result, the calculated ω is not optimal and the update of Vk(i)s in equation (10), does not enhance the acceleration of the algorithm. The dynamic calculation of an optimal ω* highly depends on h and l and if the modification is not adopted, it is likely that modified SVIA will not result in convergence. Note that the modification is essential in cases of large-scale MDPs, where I is vast and numerous “ties” occur.

4.4.2. Minimum Ratio Criterion Value iteration Algorithm

Herzberg and Yechiali (1991) [4] refined the idea of calculating a DRF only based on the “important” states h and l in order to update the values of Vk at the end of each iteration k using equations (10) - (11). In iteration k, the proposed DRF is calculated after the analysis of the values δk+1i and it is used to update the values of Vk using equation (10). Moreover, separate treatment is provided for MDPs and SMDPs. If only the states h and l are considered to acquire knowledge on the future values of  Vk+1i in the next iteration k+1, modified SVIA may not result in the optimal calculation of a DRF ω* in certain iterations. To overcome this difficulty, the variable gki is introduced:

gki=j∈IpijRi∙δkj, i ϵ I 					                      (12)

Based on gki that represents the future differences δk+1i if the same policy Ri is adopted ∀ i ∈I in iteration k+1, another variable αki is defined  as:

αki=gki-δki, i ϵ I							          (13)

The analysis continues with the definition of the Minimum Ratio Criterion (MRC). The objective is to find the optimal ω*, that will reduce the term:

Mω=π1(ω)π2(ω) ,  π2(ω)>0 							          (14)

where π1(ω) and π2(ω) represent the future maximum and minimum difference δk+1h and δk+1l respectively.

δk+1h=π1ω=maxi ϵ Iδki+ω∙αki   				           (15)

δk+1l=π2ω=mini ϵ Iδki+ω∙αki  				           (16)

ω1* and ω2* denote the values for which the minimum and the maximum ω* is obtained, from π1(ω) and π2(ω) respectively.

π1ω1*=minωπ1(ω)		     					          (17)

π2ω2*=maxωπ2(ω)		   					          (18)

with initial values set as: π10=δk(l) and π20=δk(h).

Taking advantage of the fact that π1(ω) and π2ω are piecewise linear and convex (or concave) functions, it suffices to search over their endpoints to find an optimal ω* that minimizes Mω. The MRC produces for ascending values of ω, two piecewise linear convex functions for  π1(ω) and π2ω respectively one after the other. Each of the breakpoints of these functions is produced for different increasing ω values. MDC starts searching the breakpoints of π2(ω) starting with ω=0, until an optimal ω* is found that minimizes M(ω). If the ω of a breakpoint of π2(ω), results in a larger value for M(ω) than the previous calculated values, the search is proved futile. Then MDC traverses on π1(ω) and continues searching its line for an ω*, starting from the first breakpoint (thus ω’s value is reduced). The procedure is repeated until an optimal ω* is found, remarking that before traversing from one line to the other, problems π1(ω) and π2ω are updated. The traverse from one problem to the other and the update of the problems is succeeded by multiplying δki and αki by -1, taking advantage of the duality between  the two problems. The latter is satisfied after the essential remark that the Minmax problem is the “Mirror Reflection” of the Maxmin problem. MRC iterations denoted by kMRC, ∀ k in fact indicate the number of examined breakpoints or the number of ω values found in every k. This thorough search to define ω* for each k, yields a powerful algorithm which applies relaxation on the values of Vki and reduces the total computational effort until convergence.

4.4.3. Minimum Difference Criterion Value iteration Algorithm

Herzberg and Yechiali (1994) [5] propose a faster and simpler criterion than MRC, which is called Minimum Difference Criterion (MDC). It is applicable for MDPs and SMDPs, considering different scheme variations of equation (1). The objective is to reduce in each iteration k, the minimum difference Dω of the values of π1(ω) and π2ω. MDC is calculated by the following equation using equations 15 - (18).

Dω = π1(ω)-π2ω							          (19)

The values of Vk at the end of each iteration k, are no more updated using equation (10), but based on the calculated future differences gki as follows:

Vki=Vk-1i+ω∙gki						                      (20)

Faster convergence is achieved by reducing the number of needed iterations that MDCVIA performs, on the expense of a computational effort per iteration higher than SVIA. This is due to the fact that when using the MDC together with SVIA, in each iteration one also has to compute the vectors gki and αki. The analysis concerning the calculation of π1(ω) and π2ω that takes advantage of the duality between the corresponding Minmax and Maxmin problems for MRC, is also applied in the MDC case. To conclude, there are cases that MDCVIA is proved to require almost the same time to converge within less iterations compared to SVIA. Thus, the total time needed to find an optimal policy Ri, ∀ i ∈I for an undiscounted MDP using MDCVIA, is lower than SVIA.

Similarly to the case of modified VIA and MRCVIA, MDCVIA modifies SVIA’s 4th step in every iteration k in order to calculate a DRF and adds a 5th step to update the value of every Vki, i∈I, before proceeding to the next iteration k+1. Thus the 4th step is MDC which contains 5 sub-steps that are repeated kMDC times, until an optimal DRF is calculated. In these kMDC iterations the MDC performs a search so as to find the optimal ω*, that minimizes D(ω) in iteration k+1. The optimal ω* is found in a breakpoint of π1(ω) or π2ω, by traversing from one problem to the other. Different optimal values ω* are produced based on the MDC proceeding from one iteration k to another, in order to efficiently approximate - after the update in step 5 - the values of Vk+1i, i∈I. In this way a successful one step look-ahead analysis is performed for every k. The steps of MDCVIA are summarized below:

# Step 0 (initialization). Fix

V0i, i∈I, to satisfy: 0≤V0i≤mina ∈ Ai{Cia} 		             and set  k=1.

# Step 1 (value improvement step). Compute

Vki=mina∈Ai{Cia+j∈Ipija∙Vk-1(j)}, i∈I 		            (1)

# Step 2 (apply bounds on the minimal costs).Compute

mk=δk(l)=min i∈I{Vki-Vk-1i}, i∈I				(3)

Mk=δk(h)=max i∈I{Vki-Vk-1i}, i∈I				(4)

# Step 3 (stopping test). If

0≤Mk-mk≤ε∙mk 							(7) Stop.

# Step 4 (dynamic relaxation factor calculation). Compute:

gki=j∈IpijRi∙δkj, i ϵ I 			      		          (12)

αki=gki-δki, i ϵ I						          (13)

Step 4.0 (DRF initialization). Set ω*=0, δ=Mk. If state h is not unique select the state with the highest value of αk∙. Set α=αk(h) and kMDC=1.

Step 4.1 Compute					 ω1=mini:αki>aδ-δkiαki-a>0, b is the state where the minimum is attained.

Step 4.2 Compute γ=αk(r), 						where  r is the state corresponding to mini ∈  I{(δki+ω1∙αki}.

Step 4.3 (DRF stopping test) If α≤γ and αn(b)≥γ,			  set ω*=ω*+ ω1 and stop.							    If αkb<γ, go to step 4.4. 							    If  α>γ, go to step 4.5.

Step 4.4 (Search DRF in π1ω1). 					        Update δki=δki+ω1∙αki, i∈I and ω*=ω*+ ω1.                      Set δ=δkb,  α=αk(b).						              Set kMDC=kMDC+1 and go to Step 4.1.

Step 4.5 (Search DRF in π2ω1). 				                   Update δki=-δki, α=-αki, i∈I. 			     	    Compute mk=δkl=max i ∈ Iδki, α=αk(l).                                    	   Set kMDC=kMDC+1 and go to Step 4.1.

Step 5 (Apply relaxation on Vkis). Update					 Vki=Vki+ω*∙gki, i∈I				                      (20)  set k=k+1 and go to step 1.

When MDCVIA case is used for cost minimization problems, the lower bound mk no more produces monotonic and geometrical sequences as in the SVIA case. Moreover mk is characterized by periodicity issues. The upper bound Mk remains robust in this case, yielding monotonic sequences. The result of the bounds synthesis is the minimal infinite horizon expected average cost Gk*, which inherits the non-monotonous behavior from mk. By violating the monotonicity properties, MDCVIA manages to converge faster than SVIA. The example case on page 20 is solved via MDCVIA within k=72 and tCPU=4 sec, which is a better performance compared to SVIA. Although convergence is almost achieved in k=30, MDCVIA needs several iterations until it finds the optimal policy Ri, i ∈I. The latter indicates the possibility of a faster solution to exist. The diagrams regarding convergence and the Gk*, follow:

Figure 3: Non-monotonic convergence of δk(i)s (left) and Gk* (right).

MDC manages to update successfully the values of Vki, ∀ i, i∈I in the example case, by calculating a DRF that fluctuates between 0.5995 and 3.7644. In general, a DRF takes values around 1 and its value sporadically reaches 2 or 3 in single iterations. In the example case, it ranges around 1.5 - 2, due to the small state space I and corresponding admissible actions Ai, ∀ i ϵ I. The search effort that MDC applied through an iteration of MDCVIA, ranges from 2 till 11 kMDC.

Figure 4: Values ω* (left) and number of  kMDCs (right) per k respectively.

In the investigated instances, the optimal ω* is usually found in one of the breakpoints of π2(ω) (regarding the optimal prediction of mk+1 in iteration k+1), while π1ω is used once in a while to re-tune the search. The search starts with the breakpoints of π2ω. When Dω is lower than the Dω calculated in a previous breakpoint, the search stops among the breakpoints of π2ω and continues with the breakpoints of π1ω. It seems that the search never remains in π1ω for more than one of the kMDC iterations. Thus, π1ω is used to stop the unsuccessful search over the line produced by π2(ω) and after providing a single ω regarding its first breakpoint, it traverses to π2ω again to restart a similar search on - an updated - π2ω, until  ω* is found. As a result, the opposite behavior of the MDC is expected for reward maximization problems, with the sporadic intervention of a low value ω - this time ω corresponds to a breakpoint found on the line produced by π2(ω) - in order to interrupt and restart a better search over the updated line produced from π1ω values. MDC provides special treatment in a case of a “tie” amongst one or more δk(i), i ∈I, in step 4.0. A direct result of the applied relaxation combined with the one step look-ahead analysis, is that less “ties” are encountered. In the example case, the highest number of iterations kMDC that MDC performed per SVIA iteration k,  occurred in iteration k=11. MDC started searching ω* in the breakpoints of π2(ω) in iterations kMDC=1,2 and traversed to the first breakpoint of the updated π1(ω) in kMDC=3 providing a low value ω. Then MDC traversed again in π2(ω) to continue the search for the rest of kMDC∈[4,…,11], until the optimal ω*=1.275 was found in kMDC=11. Moreover, the usage of MDCVIA yields 113 “ties” in total regarding mk, while no “ties” were observed regarding Mk. This reduced number of “ties” by 93.7% compared to SVIA for the same state space I, indicates the obstacle that “ties” put towards fast convergence.

Figure 5: 11 Different ω values found in k=11, for 11 corresponding kMDCs (left) and number of “ties” found per k (right).

To conclude, the computational performance of MDCVIA is compared next to the one of SVIA. The Computational Effort per Iteration k (CEI) needed by SVIA, mainly depends on the structure of the one step transition probability matrix. Thus, for a fully dense matrix the CEI is A∙I2, with A denoting the average number of allowable actions per state i. In real-life practical problems matrices are sparse and the CEI is M∙A∙I (Herzberg et al. (1994) [5]), if the average one step transitions are denoted with M, M≪I. This is the total CEI required by SVIA, to compute the values of Vki, i∈I. In the MDCVIA case, gki and ω are also computed, in addition to Vki. The CEI that MDCVIA needs to compute gki, is M∙I. The corresponding CEI needed for the optimal ω*, ranges between 4∙I and 12∙I. As a result, the additional CEI ranges between (M+4)∙I and (M+12)∙I. The total CEI of MDCVIA fluctuates between (M∙A+1+4)∙I and (M∙A+1+12)∙I. Although the CEI of MDCVIA is bigger than the CEI of SVIA, MDCVIA converges faster because it needs less iterations k. The saved CEI for reducing an iteration is M∙A∙I. Thus, the algorithm is beneficial for problems where A is big and CEI is M∙A∙I>(M+4)∙I.

4.4.4. K-step Minimum Difference Criterion Value iteration Algorithm

Herzberg and Yechiali (1996) [6] in an attempt to further improve the performance of MDCVIA, integrate the idea of relaxation with the idea of K value oriented steps in the future. A mixture of both techniques is used to find optimal policies for MDPs and SMPDs. Moreover, the undiscounted case and different scheme variations of equation (1) are considered. In this MDC variant several (K) steps are performed in an iteration of MDCVIA, resulting in value functions Vki that are updated K times within an iteration k. Thus, the future estimators VK,ki of K-step MDCVIA, are acquired through the relations:

VK,ki=Vki+m=1Kωm,k ∙ gm,n(i), i ϵ I, K=1…KMAX		          (21)

δK,ki=δk-1i+ ωm,k ∙ gK,ni-δk-1i, i ϵ I, K=1…KMAX 	          22

where V0,ki=Vki,  δ0,ki=δki and

gK,ki=j∈IpijRi∙δk-1j, i ϵ I, K=1…KMAX  			          23

MDC may increase significantly the CEI, because K updates of Vki are performed per iteration k. These K updates in an iteration of the modified SVIA, behave like a variant of Policy Iteration that is known as Modified Policy Iteration (MPI). In every iteration k, the values of Vki are updated under the same policy Ri, resulting in VK,kis that are state and not action dependent.  Thus, the proposed algorithm uses the concept of relaxation and value-oriented in a unified framework, in order to acquire insight on the Kth future step. This is the reason why, K-step MDCVIA is categorized amongst the Fathoming and Relaxation Criteria. The wise use of the K-step MDC is remarked, in order to ensure that the DRF ωm,k is not calculated in every step K, but only in selected steps. Else the performance of K-step MDCVIA, may be worst compared to SVIA. Herzberg and Yechiali (1996) [6] propose several modifications and rules on how to fix an updating schedule of the future estimators VK,ki.

To conclude, SVIA is the most famous among numerous algorithms, that are able to find an optimal policy for a MDP. SVIA can be accelerated significantly by applying relaxation at the value functions Vki, via a relaxation factor ω. A plethora of accelerated SVIA variants are used, to solve MDPs and SMDPs for discounted and undiscounted cases. The difference between these variants, is the criterion they use to calculate an optimal relaxation factor ω*. In practice, accelerated algorithms are used to find optimal policies for large scales MDPs, where the state space I contains millions states i. When SELSP is modeled as a MDP, large scale MDPs are certain to occur. The formulation of the MDP model, follows in the next Chapter 5.

5. Mathematical Model for SELSP

The dynamic scheduling problem of a single machine that produces several grades of a product, can be formulated as a discrete-time undiscounted MDP or as a discounted SMDP (Liberopoulos et al. (2009) [11]). In this survey the first approach is adopted, but the cost formulation can also used for SMDPs. The assumptions adopted to formulate the problem are listed below:

  - Continuous production environment for SELSP

  - Intermediate grade

  - Periodic review control policy

  - Global lot sizing policy

  - Dynamic sequencing

  - Infinite time horizon

  - Medium-term scheduling

  - Discrete time MDP

  - Weak Unichain Assumption

  - Stationary demands

The notation that is used to calculate the incurred cost Cia in each iteration k for every decision a, for every state i of the MDP follows:

# Parameters

n: Grades of products, n=1,…,N-1,N

P: Production rate, constant for all grades and periods

X: Capacity of the warehouse

Dn : Random bounded demands for each grade n

SPC: Spill-over cost per unit of excess product

CLn: Lost sales cost per unit of unsatisfied demand per product n

SWC: Switchover cost per setup change

# States & Actions

i: States of the system at the beginning of each period, where

i≡(s,x1,…,xN),  s ϵ Ai, n=1,..,N, xn ∈ Z ,            				 i ϵ I,  I=[0,1,…]

s: The current grade that the facility produces (current setup)

xn: Inventory level of grade n at the beginning of a period ∀ grade, 			xn ∈ Z

a: Decision on which grade the machine will produce ∀ state i, a ϵ Ai, where

Ai: Set of the allowable decisions ∀ state i, if s is the current setup, 			Ai ϵ A,A⊆[1,…,N]

A: Set of the allowable decisions for all states

prodi: Amount added to the FG buffer in state i

s.t. Constraints (27) – (28)

Ia: Indicator function, if a is true Ia=1, else Ia=0

j: State of the system at the beginning of the next period if decision a is			taken in state i

j≡s',x1',…,xN'=f(i,a), where

s'=a and xn'=maxi ϵ I,∀n{0,xn+prod(i)∙In=s-Dn}	          (24)

# Cost Function

Cia: Total cost incurred for decision a and state i ϵ I

Cia=SWC∙Ia≠s+SPC∙P-prodi+nCLn∙maxi ϵ I,∀n0,Dn-xn-prodi∙In=s                 (25)

# Constraints

FG Inventory Constraint

0≤nxn≤X, n=1,..,N 						          (26)

State Production Constraint

prodi=minP,X-nxn  	           				          27

Machine Production Constraint

P= nE{Dn} , P ϵ Z 		          				          (28)

Equation (24) indicates in which state j the MDP will jump to from state i, after satisfying or not every incoming demand for every grade n. Inventory constraints (26) are applied, to set the individual grades’ allowable inventory levels with respect to X. To define the amount of the grade produced in the machine in state i equation (27) is used. Equation (28), is used to balance the maximum production the machine produces in each period k, with the sum of the expected demands for every individual grade. A variable P is not able to model efficiently the described process and produces instabilities. Liberopoulos et al. (2010) [10] used the above assumption to model a real-life practical problem in a multi-grade PET resin industry.

The values of Cia,i∈I,a∈Ai are always positive, since they are a summation of the individual (positive) costs SPC, SWC and CLn and depend on the current state i and the decision taken a. This dependency results from the dynamic environment, due to the fact that for states i with high inventory level of at least one grade n spill over-costs occur. Respectively, lost demands may occur in those states i, with low inventory level of at least one grade n. Thus states i where SPC and CLn occur can be predefined by calculating them from equations (27) and (24) respectively. Those states i where CLn may occur for one or more grades n are calculated, making use of the second term inside the max{∙} of eq. (24). The term calculates the inventory levels in a state j where the MDP jumps to after one-step transitions from state i, under the incoming stationary demands Dn. The corresponding states where SPC may occur, are the ones that after using equation 27 they satisfy prodi<P. A similar idea cannot be applied safely regarding the SWC and corresponding states i, where the occurrence of a switch over is certain to take place. In order to acquire full insight on the cost function Cia considering also SWC, equation (25) has to be calculated for every possible decisions a, a∈Ai and for every state i. After this thorough analysis of the MDP, it becomes clear that the cost Cia incurred in a state i, for a decision a, remains unchanged throughout the iterations performed by SVIA.

According to the above discussion those cost (SPC, CLn) “sensitive” states i, can be grouped into classes according to the inventory levels xn of their individual grades. The inventory levels are presented graphically, irrelevantly of the grade currently produced in the machine and the decision a chosen. Two example cases are introduced, that consider a machine working with P=2 and N=2, for X=15 and X=30 respectively. Then the following figures can be easily drawn using equations (27) and (24), to illustrate a representation of the “dangerous” inventory levels (x1, x2) of a warehouse. Thus, the red color indicates the area with all the allowable combinations of inventory levels xn. The diagonal line is drawn after using equation (26), to indicate the maximum capacity of the warehouse in each case. In both cases, the areas prone to cost are indicated with white lines. The outlined area parallel to the diagonal line, denotes inventory levels xn where SPC may occur. The outlined rectangular areas parallel to the axes indicate respectively for the two grades, the inventory levels xn where LSC may occur. These areas are bounded from the maximum demand occurred for each grade respectively. In total, the graphs describe the warehouse cost behavior. SPC occur when the warehouse is almost full and LSC occur when there is a lack of a grade of a product.

Figure 6: Schematic illustration of state classes in which SPC and/or CLn is guaranteed to occur w.r.t. points (x1,x2), when X=30 (left) and X=15 (right).

This rather simple remark demonstrates the way that states prone to cost can be identified based on their inventory levels. However, in the next Chapter we show that identifying these states is very beneficial in designing a heuristic.

6. Heuristics

In this Chapter, a 2-Grade Action Elimination (2-GAE) heuristic is presented, which is based on graphically represented SELSP solutions. Before proceeding with the analysis of the heuristic procedure in Section 6.2, an introduction is provided to the AE concept and related techniques in Section 6.1.

6.1 Action Elimination

Action Elimination (AE) is one of the most wide-spread techniques used to enhance the convergence rate of SVIA. The main concept of AE is to find those actions a, a ϵ Ai,  ∀ i ϵ I, that are proved not to be optimal (sub-optimal) after an AE test or according to various  criteria. If a sub-optimal action a, a ϵ Ai is calculated, then  it can safely disregarded in future iterations of the algorithm. The objective of the method, is to reduce the number of allowable actions a within every set of actions Ai that corresponds to every state i and as a consequence to reduce the state space I. Then the reduced state space I will ensure a faster convergence of SVIA.

Numerous AE techniques have been developed for enhancing SVIA, LP and PIA. Some of them apply tests in the end of each iteration k based on the value functions Vk(i) or the bounds mk and Mk, to identify and eliminate sub-optimal actions. In other AE techniques, coefficients of the transition probability matrix are calculated in order to provide tighter bounds. A part of the existing techniques perform permanent AE, while in temporary AE techniques a sub-optimal action a may appear again in future iterations, re-entering in this way the set Ai,  i ϵ I. Similarly to the literature regarding relaxation on value functions Vk(i), the majority of literature on AE focuses on the discounted MDPs. Jaber (2008) [7] provides a review on the AE techniques.

6.2. 2-Grade SELSP Action Elimination

The heuristic procedure described in this Section is based on Hatzikonstantinou’s [3] (2009) graphical representations of the optimal policy for SELSP. In this thesis, these representations are produced at the end of an iteration of SVIA in order to identify optimal actions a.The optimal policy Ri can be depicted in a graph, with respect to the individual inventory levels of each grade. As a result, areas are formed that contain similar characteristics. The objective of the heuristic developed in this thesis, is to forecast the optimal decision a for a number of states i, that belong to one of the resulting areas. Thus, the other suboptimal actions of theses states are disregarded, resulting in a heuristic that accelerates the performance of SVIA and its variants. Firstly, the way to illustrate an optimal policy is presented and the heuristic procedure follows. The discussion regards 2-Grade SELSPs and can be extended for SELSPs that consider more grades of a product.

In order to illustrate the optimal policy Ri, it is decomposed with respect to each of the two grades, to produce two 2-dimensional vectors R1(x1,x2) and R2(x1,x2). Consequently, R1(x1,x2)  contains those actions a chosen, when grade 1 is produced and R2(x1,x2) the actions a chosen, when grade 2 is produced. The components of the vectors are the inventory levels x1 and x2. Considering that each state i, i∈I is defined as i=(s,x1,…,xN)  and N=2, the two vectors R1(x1,x2)  and R2(x1,x2) contain information on which decision a is taken for s=1 and s=2 respectively, for every point (x1,x2). Thus, the optimal policy Ri can be decomposed and illustrated for s=1 and s=2. The green color in each graph, indicates the inventory levels where the action to produce grade 1 is decided and the red color indicates the corresponding inventory levels for the action to produce grade 2.

Figure 7: Decomposed optimal policy for(x1,x2) w.r.t the produced grades s=1 (left) and s=2 (right), for Case 1 and X=40.

After producing the two graphs, they are synthesized in a single graph by choosing every possible combination of decisions a, for every point (x1,x2) ∈ R1(x1,x2) and R2(x1,x2) .To simplify that, for each one of the 4 possible decision combinations that may occur - in a 2-Grade SELSP - a different region is illustrated in a graph. This done, by combining the decomposed policies for every inventory level combination (x1,x2). The produced regions in the final graph vary from 3 to 4, according to the number of decision combinations occurring for each (x1,x2), which depends on the cost settings of each case. Graphically, 4 regions occur when the red region from the left graph in Figure 7, overlaps with the green from the right graph for some points (x1,x2). The regions which represent the final decision combinations are:

R1(x1,x2)=R2(x1,x2)=1                                           region tangential to x2

R1(x1,x2)=R2(x1,x2)=2				     region tangential to x1

R1(x1,x2)=1 && R2(x1,x2)=2		                     upper middle region

R1(x1,x2)=2 && R2(x1,x2)=1		                     lower middle region

The final graph depicts the optimal policy Ri found and can be used to schedule the single machine, considering only the current inventory levels (x1,x2). Thus, when (x1,x2) belongs to the region tangential to the axis x1 or x2, production is switched to grade 1 or 2, respectively. When (x1,x2) belongs to the upper or lower middle region, production remains the same or changes to the other grade, respectively. It is observed that for high values of SWC and X, the lower middle region is absorbed by the dominating upper middle region. This is natural, since in case of a high SWC or X, the optimal policy indicates to continue producing the same grade which is currently under production. In such a case, 3 regions occur instead of 4.

Figure 8: Optimal policies Ri with 4 regions (left) and with 3 regions (right).

The policy is formed gradually over the k iterations of SVIA, until the final optimal policy Ri is found. The same happens with the graphs, where the shapes of the regions change over the k iterations of SVIA. After experimentation, the tangential to the axes regions shrink over the evolution of SVIA. The upper middle region tends to grow against the “sensitive” tangential regions. The sensitivity of these regions has been already discussed in Chapter 5. The result of the behavior of the regions is that the upper middle region is stable and produces the optimal policy from the first iterations of the algorithm. The latter attribute is considered in the heuristic procedure, to acquire knowledge of the graph at the next iteration k+1. The example case is considered this time with SWC=10, to illustrate the evolution of the policy and the dominance of the target region.

Figure 9: (Premature) Optimal policies Ri found, in k=4 (left) and k=20  (right) solved via 2-GAE.

The presentation of the 2-GAE follows. Based on the observations of the regions, a graph of the optimal policy Ri is produced at the end of an iteration k. Such an action provides an indication on the future policy, which can be used in order to adopt partially that policy Ri. Hence, AE is not applied by 2-GAE for every (x1,x2), but only to those which  belong to the upper middle region. A search procedure is used to sweep every point (x1,x2),  in order to allocate the dynamic thresholds of the target region (upper middle region). Finally, the optimal decisions a found in the target region are adopted in the next iteration k+1, for every state i with inventory levels (x1,x2) belonging to that region. For these states in the next iteration, SVIA computes the value Vk+1(i) for only one decision a. In this way, AE based on graphs of the policy of the previous iteration, is performed on a subset of the state space I. The 2-GAE heuristic, does not exactly performs AE by finding sub-optimal actions, but takes advantage of the stable target region throughout the evolution of the algorithm. Moreover, this reverse application of AE - that in fact is a partial policy adoption - performs better when the target region is wide, because policies are adopted for more points (x1,x2). Wide target regions are observed for big values X, which yields big state spaces I, increasing the performance of the method. In order to cope with the increased complexity of regions and the volatility of the corresponding policies when X is small, some modifications are required. The dynamic thresholds found in every k are relaxed by 3 units, at the expense of the target region. If tighter bounds are selected the results are catastrophic for the optimal policy and corresponding costs.

The procedure can be added as the last 6th step within SVIA or any of its variants, in order to solve the 2-Grade SELSP. The steps of 2-GAE are summarized below:

Step 0 (Graph the optimal policy)

Decompose the optimal policy Ri into R1(x1,x2) and R2(x1,x2) w.r.t s1 and s2.

Combine R1(x1,x2) and R2(x1,x2) into a single graph.

Step 1 (Threshold relaxation)

If d(xn)≤3, the target region does not contain (x1,x2), where

d(xn): Distance between a threshold of the target region and respective xn, n=1,2, for every inventory level (x1,x2) that belongs to the target region.

Step 2 (Action Elimination step)

Choose the decisions a found in the target region, to calculate Vk+1(i) for all the states i with (x1,x2) belonging to the target region.

To conclude, 2-GAE is subject to several modifications according to the cost structure of the 2-Grade SELSP instance that is considered. The dynamic thresholds can be relaxed in different ways and actions can be chosen from other regions as well. In this thesis a universal modification is proposed that is able to perform AE efficiently irrelevantly of the parameters of a SELSP instance. The performance of 2-GAE is demonstrated in the next Chapter 7, that contains the numerical experiments.

7. Numerical Experiments

In this Chapter, comparisons are conducted between the computational time and the number of iterations of SVIA, MDCVIA and MDCVIA enhanced with 2-GAE. The algorithms are tested on different 2, 3, and 4-Grade SELSP cases. The data description is given in Section 7.1. In Section 7.2. the influence of  the initial state on SELSP is presented. The Chapter ends with the results presented in Section 7.3.

7.1. Data description

For the 2-Grade SELSP, the algorithms were tested on 10 basic cases. Each basic case corresponds to a different cost combination (Table 1), with the rest of the parameters P=5, ε=10-3 remaining fixed. It is assumed LC1=LC2 for every case. Demand distributions for every product n, are shown in Table 2. For both grades, the demands are distributed following an upper triangular distribution. The highest probability for every grade, corresponds to the demand that is equal to the mean of the grade demands. Finally, variations of each basic Case 1 - 10 are considered, where the capacities of the warehouse varies for X=[40, 60, 80, 100].

| Case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SWC | 1 | 1 | 2 | 5 | 5 | 2 | 10 | 10 | 1 | 1 |
| SPC | 5 | 10 | 5 | 10 | 1 | 10 | 1 | 5 | 10 | 5 |
| LCn | 5 | 10 | 5 | 1 | 10 | 10 | 1 | 10 | 5 | 10 |

Table 2: Cases 1 - 10  w.r.t. different cost combinations.

The different cost combinations are considered in Cases 1 - 10, in order to investigate the impact on the different algorithms. This is easily investigated for 2-SELSP Cases where the computational effort the algorithms need is small, but not for 3 and 4-SELSP Cases.

| n Dn | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.1 | 0.15 | 0.15 | 0.2 | 0.15 | 0.15 | 0.1 |
| 2 | 0.15 | 0.15 | 0.4 | 0.15 | 0.15 | 0 | 0 |

Table 3: Probability distributions of Dn, for Cases 1 - 10, for the two grades.

In all cases, the demands and the demand distributions are chosen in this way, in order to reproduce a stochastic environment, where different one step transitions occur. The uncertainty is achieved by setting different demand distributions, for each grade. Also, the range of demand must be different between grades.

In order to investigate the sensitivity of the 2-Grade SELSP for different demand distributions, Cases 11 - 17 are also considered. In these seven Cases, the parameters are X=80, SPC=1, LCn=5, , LC1=LC2, SWC=1, P=5 and ε=0.05 for the demand distributions of Table 4. The demand distributions considered are: the equiprobable, the upper and the down triangular, the ascending and the descending. The triangular distributions are also symmetrical and in all Cases the probabilities range between 0.1 and 0.27. In Case 16 and 17, we consider respectively descending and ascending demand distributions, but the allowable range for the probabilities is from 0.003 to 0.505.

| Case | n Dn | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11 | 1 | 0.143 | 0.143 | 0.143 | 0.143 | 0.143 | 0.1435 | 0.143 |
|  | 2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0 | 0 |
| 12 | 1 | 0.1 | 0.1333 | 0.1666 | 0.2 | 0.1666 | 0.1333 | 0.1 |
|  | 2 | 0.1 | 0.225 | 0.35 | 0.225 | 0.1 | 0 | 0 |
| 13 | 1 | 0.175 | 0.15 | 0.125 | 0.1 | 0.125 | 0.15 | 0.175 |
|  | 2 | 0.27 | 0.18 | 0.1 | 0.18 | 0.27 | 0 | 0 |
| 14 | 1 | 0.19 | 0.18 | 0.16 | 0.14 | 0.12 | 0.11 | 0.1 |
|  | 2 | 0.3 | 0.25 | 0.2 | 0.15 | 0.1 | 0 | 0 |
| 15 | 1 | 0.1 | 0.11 | 0.12 | 0.14 | 0.16 | 0.18 | 0.19 |
|  | 2 | 0.1 | 0.15 | 0.2 | 0.25 | 0.3 | 0 | 0 |
| 16 | 1 | 0.32 | 0.215 | 0.15 | 0.14 | 0.111 | 0.061 | 0.003 |
|  | 2 | 0.505 | 0.246 | 0.107 | 0.082 | 0.06 | 0 | 0 |
| 17 | 1 | 0.003 | 0.061 | 0.111 | 0.14 | 0.15 | 0.215 | 0.32 |
|  | 2 | 0.06 | 0.082 | 0.107 | 0.246 | 0.505 | 0 | 0 |

Table 4: Probability distributions of Dn for Cases 11 - 17, for the two grades.

For the 3-Grade SELSP, 6 variations of the basic Case 18 are considered, for increasing values of X. The parameters of Case 18 are: SPC=LCn=2, SWC=1, LC1=LC2, P=6 and ε=10-2. The comparisons are conducted for X=[15, 20, 30, 40, 50, 60]. Demands and corresponding probabilities for every product n, are given in Table 5. The demand distributions come from a real life problem, where probabilities descend as the demand grows (Hatzikonstantinou (2009) [3]). The highest probability is set for demand equal to 0, for grades 1 and 2. For grade 3, the highest probability corresponds to demand equal to 2.

| n Dn | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0,1676 | 0,1429 | 0,3214 | 0,1538 | 0,1016 | 0,0604 | 0,0247 | 0,011 | 0,0137 | 0,0027 | 0 |
| 2 | 0,5 | 0,1648 | 0,1071 | 0,0824 | 0,0604 | 0,0302 | 0,022 | 0,0137 | 0,0027 | 0,011 | 0,0055 |
| 3 | 0,1519 | 0,2652 | 0,2956 | 0,0718 | 0,0663 | 0,0525 | 0,0442 | 0,0138 | 0,0276 | 0,0028 | 0,0083 |

Table 5: Probability distributions of  Dn, for the three grades.

For the 4-Grade SELSP, 4 variations of the basic Case 19 are used. It is assumed: SWC=SPC=LCn=1, LC1=LC2, P=6, ε=10-2, with the corresponding experiments conducted for X=[10, 15, 20, 25]. The range of the demands is smaller compared to Case 18. Demands and corresponding probabilities for every product n, are given in Table 6. Similarly to the Cases 1 - 10, and 12, the demands are distributed following an upper triangular distribution, for the four grades. This time, the highest probability for every grade is set around the mean value of demands. This leads to asymmetric triangular distributions.

| n Dn | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| 1 | 0,25 | 0,5 | 0,25 | 0 |
| 2 | 0,05 | 0,2 | 0,45 | 0,3 |
| 3 | 0,05 | 0,2 | 0,45 | 0,3 |
| 4 | 0,25 | 0,5 | 0,25 | 0 |

Table 6: Probability distributions of Dn, for the four grades

7.2. Influence of the initial state on SELSP

As aforementioned and proved in Section 4.3, the optimal policies found under the WUA, do not depend on initial states. This is opposed to the model proposed in the relevant work of Liberopoulos et al (2009) [11]. Nevertheless the independence of initial states, the model finds the same results with slight divergence for some Cases. The data for the Cases 1 - 10, 18 and 19 considered in this survey, are found in the works of Liberopoulos et al. (2009) [11] and Hatzikonstantinou (2009) [3]. Detailed results of these comparisons regarding the Cases 1 - 10, can be found in Appendix. In the relevant matrix the values marked with a [*], indicate the found values are slightly larger, when compared to the results of the model that was solved via SVIA in the paper of Liberopoulos et al (2009) [11]. The majority of the rest of the values were found smaller and some of them equal to those in [11], regarding k and Gk*. Finally the optimal policies that were found slightly different - but still optimal - are denoted as ε-optimal.

7.3. Algorithm Performance Comparisons

In this Section the performance of SVIA, MDCVIA and MDCVIA enhanced with 2-GAE is presented and compared, on Cases 1 - 19 for variable X values. The increasing X values on the same Case variant, increase the state space I, the number of iterations k and the computational effort denoted by tCPU. Thus, in every comparison on a single Case variant, the tCPU (in seconds) and k of every algorithm is compared. Moreover, the different cost combinations in Cases 1 - 10, allow to compare the performance of 2-GAE for ascending X values. In subsection 7.3.1., the performance of all algorithms for every variant of Cases  1 - 10, is compared. The performance of 2-GAE for different X values of Cases  1 - 10, is also presented. Finally, 7.3.1 concludes with the comparison of SELSP for different demand distributions, for Cases 11 - 17. The comparison of MDCVIA’s performance against the one of SVIA follows in 7.3.2., for Cases 18 - 19.

When one of the Cases is solved several times via MATLAB, the tCPU varies slightly. For small X regarding the 2-Grade cases 1 - 10, the variation of tCPU is less than 0.1 sec. As the grades and the state space I grow, this variation of tCPU also grows, but is always a small proportion of  tCPU. Moreover, for several repetitions of a single experiment, the resulting tCPU constantly ranges around the same value. Thus, the experiments presented in this Section are repeated one time, leading to safe results.

7.3.1. 2-Grade SELSP

MDCVIA and MDCVIA enhanced with 2-GAE are compared against SVIA, on 40 Cases regarding the 2-Grade SELSP. The results of the experiments are presented in Figures 10-14. In each figure, the performance of SVIA, MDCVIA and MDCVIA enhanced with 2-GAE is compared, on the Cases 1-10 for every X, X=[40, 60, 80, 100]. MDCVIA (red line) always outperforms SVIA (blue line) and in its turn MDCVIA enhanced with 2-GAE (green line) always outperforms MDCVIA, regarding both tCPU and the number of iterations k. The performance of the two methods, encouragingly increases proportionally to the growth of the state space I. MDCVIA improves the tCPU that SVIA needs on average by 43.87%, while when it is enhanced with 2-GAE the tCPU is improved on average by 47,6%. The enhancement of both variants of MDCVIA on the tCPU of SVIA is significantly reduced in Case 7, especially when MDCVIA alone is used. The average improvement for the four X values in this Case, is only 21,17%. When MDCVIA is used along with 2-GAE the average improvement reaches up to 34,31% in Case 7. Finally, both MDCVIA variants need less iterations compared to SVIA.

Figure 10: Comparative results for X=40, regarding k (upper) and tCPU in seconds (lower).

Figure 11: Comparative results for X=60, regarding k (upper) and tCPU in seconds (lower). Figure 12: Comparative results for X=80, regarding k (upper) and tCPU in seconds (lower).  Figure 13: Comparative results for X=100, regarding k (upper) and tCPU in seconds (lower).

The performance of 2-GAE on SELSP depends on the target region and consequently on the state space I, as discussed in 6.2. The target region is the upper middle region of the graphed optimal policy. This target region depends on the cost combination of each Case. The performance of 2-GAE is measured by the ratio between the total actions α adopted and the total actions that MDCVIA would have calculated without. Note that MDCVIA computes in every iteration I∙A actions a. The ratio ranges from 5% until 30% for X=40 and X=100 respectively. The average performance for the 40 Cases is 18%.

Figure 14: Increasing performance of  2-GAE, w.r.t. increasing values of  X.

For every Case except Case 7, 2-GAE reduces tCPU of MDCVIA by the same percentage. For Case 7 the improvement is the largest. Case 7 is the only Case when using MDCVIA, that tends to require the same tCPU as when it is solved using SVIA. 2-GAE comes to overcome this MDCVIA’s weakness and accelerates the convergence to the required expected level. Case 7 seems to perform rather well when using SVIA, while showing relevant insensitivity to the other algorithms. The reason to the bizarre behavior of Case 7, is the high values of SWC compared to SPC and LCn, which yields a wide upper middle region. Thus, a conclusion is that cases with similar cost combinations to Case 7, are easy to compute via SVIA. Opposed to this ambiguous conclusion, a direct result of the comparison is the increasing performance of the two methods as the state space I becomes larger; an essential attribute when solving real life problems like SELSP, that that are governed from large-scale MDPs.

Finally, SVIA is tested on Cases 11 - 17 that regard different demand distributions, in order to investigate the dependency of SELSP on them. Indeed, the behavior of the SELSP changes against the different incoming demand patterns. Case 12 which has an upper triangular distribution, is indicated as the most computational consuming one. Moreover, the optimal policy Ri* of Case 12, yields the smallest Gk*. Thus the highest the optimal Gk*, the less tCPU is required. Regarding Cases 14 - 17, the ascending demand distributions need less tCPU, compared to the descending ones. This happens for ascending demand distributions, because there is a highest probability regarding high demand values. The latter results in high values of optimal Gk*, that need less tCPU. Concerning again Cases 14 - 17, the required tCPU is reduced in 16 and 17 that their range of probabilities is larger than the respective range of probabilities in Cases 14 and 15. The comparison of the results follows in Table 7.

| Case |  | k | tCPU | Gk* |
| --- | --- | --- | --- | --- |
| 11 |  | 931 | 138 | 0,4034 |
| 12 |  | 1197 | 180 | 0,3224 |
| 13 |  | 805 | 118 | 0,4602 |
| 14 |  | 395 | 59 | 1.9307 |
| 15 |  | 132 | 19 | 4.7764 |
| 16 |  | 175 | 26 | 4.7198 |
| 17 |  | 48 | 7.2 | 11.7599 |

Table 7: Comparative results for the Cases 11 - 17.

7.3.2. 3 and 4-Grade SELSP

Proceeding to the next conducted experiments, a real-life problem (Case 18) was considered for the 3-Grade SELSP and the simplest Case 19 for the 4-Grade SELSP. In these comparative experiments, each case was examined for increasing values of X for SVIA and MDCVIA.

When the number of iterations and tCPU of MDCVIA (red line) is compared with SVIA (green line) on Cases 18 - 19, the performance of the method encouragingly increases proportional to the growth of the state space I. In the comparisons, the maximum warehouse capacity X is reduced as the grades of a problem increase. The explanation of this setting regarding values of X, is that the number of iterations and the tCPU of SVIA is problem dependent and both rise as the state space grows. As soon as the needed tCPU in an experiment exceeded 4 hours, further investigation of values X for that Case was stopped.

Figure 15: Comparative results for Case 18 regarding k (upper) and tCPU in hours (lower), w.r.t. increasing values of  X.

Figure 16: Comparative results for Case 19 regarding k (upper) and tCPUin hours (lower), w.r.t. increasing values of  X.

MDCVIA compared to SVIA, saves up to 43.65% of tCPU in the 3-Grade SELSP and up to 25.02% for the 4-Grade SELSP case. The respective average savings in tCPU terms, are 34.81% and 17.9%. Case 18 is described by demand probabilities derived from a real-life problem, thus MDCVIA seems to perform better when solving real-life applications. The latter conclusion, combined with the already given result of increased performance when the state space grows, suggest MDCVIA for solving SELSP.

Note that the curse of dimensionality, prevents a thorough investigation of the 4-Grade SELSP case. As a result, Case 19 was studied on the two methods for a limited range of capacities X. This partially explains the lower performance of MDCVIA.

8. Conclusions and future research

In this thesis the Stochastic Economic Lot Scheduling Problem (SELSP) is addressed. SELSP is formulated as a Markov Decision Process (MDP) and due to the nature of the problem, large-scale MDPs occur. As a result the Standard Value Iteration Algorithm (SVIA) used to solve the MDP, requires a lot of computational effort to find an optimal policy for the MDP. The Minimum Difference Criterion (MDC) is efficiently used, in order to accelerate SVIA on different SELSP cases. Finally, a heuristic procedure named 2-Grade Action Elimination (2-GAE) is developed for 2-Grade SELSP instances, in order to accelerate further the solution procedure of MDCVIA.

The 2-GAE heuristic performs better, when the warehouse capacity X increases. As a result, the extension of the heuristic for the 3-Grade SELSP seems promising. Illustrating the different regions of the optimal policy for the 3-Grade SELSP has been accomplished [3]. It remains to find a way in order to locate stable regions in the graphed policy, like the upper middle region of the graphed optimal policy in the 2-Grade SELSP. Besides the AE based on graphs, there exist AE techniques that are based on cost criteria to perform effective AE on a MDP. Unlike 2-GAE, such techniques apply AE on the entire state space of the MDP and could be also used for the SELSP. An equally interesting approach, would be to solve Cases 1- 10 via K-step MDCVIA that is also enhanced with 2-GAE and Cases 11- 12 via K-step MDCVIA alone.

For SELSP Cases with many grades - thus many actions -, the approach of the K-step MDCVIA seems the most suitable. Such an experiment for a SELSP with many grades requires a lot of computational effort. The 2-GAE MDCVIA can be used within a heuristic that decomposes a multi-grade SELSP into several 2-grades SELSP that are solved via SVIA. Then, the solutions of the 2-grade SELSPs are combined, in order to construct the optimal policy. Numerous such heuristics have been developed, but the most promising approach seems to be the one proposed by Leizarowitz [9]. The elegant decomposition that is performed after thorough mathematical analysis on the structure of the MDP, provide remarkable results. Moreover, the method uses SVIA to solve the decomposed sub-problems. Thus, the solution of large scale multi-chain MDPs produced by multiple grade SELSP Cases, can be effectively accelerated.

# Bibliography

[1] Arruda, E. F., Fragoso M. D. and do Val, J. B. R. “Approximate dynamic programming via direct search in the space of value function approximations”. European Journal of Operational Research”. 211 (2011) 343-351

[2] Bellman, R. “A Markovian Decision Process”. Journal of Mathematics and Mechanics. 6/5 (1957)

[3] Hatzikonstantinou O. “Production Scheduling Optimization in a PET Resin Chemical Industry”. Ph.D. Dissertation, Department of Mechanical Engineering, University of Thessaly, (2009)

[4] Herzberg, M. and Yechiali U. “Criteria for selecting the relaxation factor of the value iteration algorithm for undiscounted Markov and semi-Markov decision processes”. Operations Research Letters. 10/4 (1991) 193-202.

[5] Herzberg, M. and Yechiali U. “Accelerating procedures of the value iteration algorithm for discounted Markov decision processes, based on a one-step look-ahead analysis”. Operations Research Letters. 42/5 (1994) 940-946.

[6] Herzberg, M., and Yechiali U. “A K-step look-ahead analysis of value iteration algorithms for Markov decision processes”. European Journal for Operational Research.88 (1996) 622-636.

[7] Jaber, N. M. A. “Accelerating Successive Approximation Algorithm via Action Elimination”.  Ph.D. Dissertation, Department of Mechanical and Industrial Engineering, University of Toronto (2008)

[8] Leachman, R. C. and Gascon A. “A heuristic scheduling policy for multi-item, single-machine production systems with time-varying, stochastic demands. Management Science. 34 (3) (1988) 377-390

[9] Leizarowitz, A. “An algorithm to identify and compute average optimal policies in Multichain Markov Decision Processes”. Mathematics of Operations Research. 28/3 (2003) 553-586

[10] Liberopoulos, G., Kozanidis G. and Hatzikonstantinou O. “Production scheduling of a multi-grade PET resin plant”. Computers and Chemical Engineering. 34 (2010) 387-400

[11] Liberopoulos, G., Pandelis D. and Hatzikonstantinou O. “The Stochastic Economic Lot Scheduling Problem for Continuous Multi-Grade Production”. 7th Conference on Stochastic Modeling of Manufacturing and Service Operations.  June 7-12, (2009) Ostuni, Italy

[12] Sox, C. R., Jackson P. L., Bowman A. and Muckstadt J., A. “A review of the economic lot sizing problem”. International Journal of Production Economics. 62/3 (1999) 181-200

[13] Sox, C. R. and Muckstadt J., A. “Optimization-based planning for the stochastic lot-sizing problem”. IIE Transactions. 29 (5) (1997) 349-357

[14] Tetsuichiro, I., Masayuki H. and Masami K. “A structured pattern matrix algorithm for multichain Markov decision processes”. Mathematical Methods of Operations Research. 66 (2007) 545-555

[15] Tijms, H. C. and Eikeboom A. M. “A simple technique in Markovian control with applications to resource allocation in communication networks”. Operations Research Letters. 5/1 (1986) 25-32

[16] Tijms, H. C. “A first course in stochastic models”. Wiley, New York, (2003) Ch. 6 233-271(ISBN: 0-471-49881-5)

[17] Winands, E. M. M., Adan, I. J. B. F. and van Houtum, G. J. “The stochastic Economic Lot Scheduling Problem: A Survey”. European Journal of Operational Research.210 (2011) 1-9

# APPENDIX

| Case | X | SVIA |  |  | MDCVIA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | k | tCPU | Gk* | k | tCPU | Gk* | Ri | tCPUSavings |
| 1 | 40 | 186 | 7,6 | 0,98 | 70 | 4,5 | 0,9796 | Optimal | 40,79% |
| 2 |  | 188 | 7,8 | 1,7411 | 67 | 4,2 | 1,7403 | Optimal | 46,15% |
| 3 |  | 179 | 7,4 | 1,1612 | 70 | 4,5 | 1,1606 | Optimal | 39,19% |
| 4 |  | 181 | 7,6 | 1,6883 | 81 | 5,2 | 1,6889* | Optimal | 31,58% |
| 5 |  | 211 | 9,1 | 1,6892 | 83 | 5,3 | 1,6879 | ε-Optimal | 41,76% |
| 6 |  | 186 | 8,1 | 1,96 | 70 | 4,5 | 1,9593 | Optimal | 44,44% |
| 7 |  | 340 | 13,8 | 1,1434* | 179 | 10,9 | 1,1433 | Optimal | 21,01% |
| 8 |  | 169* | 6,7 | 2,7074 | 72 | 4 | 2,7043 | Optimal | 40,30% |
| 9 |  | 225 | 9,2 | 1,3644* | 82 | 5,2 | 1,3651 | Optimal | 43,48% |
| 10 |  | 253 | 10 | 1,3646 | 89 | 5,5 | 1,364 | ε-Optimal | 45,00% |
| 1 | 60 | 474 | 43 | 0,6165 | 165 | 21 | 0,615 | Optimal | 51,16% |
| 2 |  | 473* | 42 | 1,0938 | 157 | 21 | 1,0935 | Optimal | 50,00% |
| 3 |  | 449* | 40 | 0,7324 | 163 | 21 | 0,7321 | Optimal | 47,50% |
| 4 |  | 437 | 39 | 1,0709 | 173 | 22 | 1,071 | Optimal | 43,59% |
| 5 |  | 516* | 47 | 1,0713 | 187 | 24 | 1,0713 | ε-Optimal | 48,94% |
| 6 |  | 474 | 43 | 1,233 | 166 | 22 | 1,2327 | Optimal | 48,84% |
| 7 |  | 369 | 32 | 0,7524 | 223 | 30 | 0,7523 | Optimal | 6,25% |
| 8 |  | 411 | 35 | 1,7234 | 160 | 21 | 1,7222 | Optimal | 40,00% |
| 9 |  | 555 | 47 | 0,8571* | 191 | 25 | 0,8572 | Optimal | 46,81% |
| 10 |  | 632 | 54 | 0,8572 | 214 | 27 | 0,8571 | Optimal | 50,00% |
| 1 | 80 | 896* | 138 | 0,4492 | 290 | 66 | 0,449 | Optimal | 52,17% |
| 2 |  | 892* | 144 | 0,7965 | 279 | 65 | 0,7964 | Optimal | 54,86% |
| 3 |  | 845* | 132 | 0,5341 | 299 | 68 | 0,534 | Optimal | 48,48% |
| 4 |  | 806 | 132 | 0,7826 | 307 | 70 | 0,7826 | ε-Optimal | 46,97% |
| 5 |  | 957* | 170 | 0,7828 | 346 | 80 | 0,7828 | Optimal | 52,94% |
| 6 |  | 896* | 141 | 0,8984 | 290 | 69 | 0,898 | Optimal | 51,06% |
| 7 |  | 408 | 60 | 0,559 | 206 | 50 | 0,559 | ε-Optimal | 16,67% |
| 8 |  | 761 | 111 | 1,2612 | 287 | 69 | 1,2611 | Optimal | 37,84% |
| 9 |  | 1032 | 154 | 0,6244 | 355 | 80 | 0,6244 | Optimal | 48,05% |
| 10 |  | 1185* | 181 | 0,6244 | 399 | 93 | 0,6244 | Optimal | 48,62% |
| 1 | 100 | 1449 | 371 | 0,3531 | 469 | 167 | 0,353 | Optimal | 54,99% |
| 2 |  | 1446 | 346 | 0,6262 | 469 | 171 | 0,6261 | Optimal | 50,58% |
| 3 |  | 1368 | 333 | 0,4199 | 492 | 172 | 0,4199 | Optimal | 48,35% |
| 4 |  | 1286 | 314 | 0,6161 | 490 | 173 | 0,6161 | Optimal | 44,90% |
| 5 |  | 1539 | 401 | 0,6162 | 563 | 200 | 0,6163* | Optimal | 50,12% |
| 6 |  | 1449 | 351 | 0,7061 | 469 | 167 | 0,706 | Optimal | 52,42% |
| 7 |  | 588 | 137 | 0,442 | 299 | 108 | 0,442 | ε-Optimal | 21,17% |
| 8 |  | 1224 | 286 | 0,9962 | 465 | 166 | 0,9937 | ε-Optimal | 41,96% |
| 9 |  | 1659 | 383 | 0,4908* | 523 | 186 | 0,4909* | Optimal | 51,44% |
| 10 |  | 1910* | 449 | 0,4908 | 565 | 204 | 0,4907 | Optimal | 54,57% |
| Mean |
| 43,87% |

| MDCVIA 2-GAE |  |  |  |  |  |  | 2-GAE |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case | X | k | tCPU | Gk* | Ri | tCPU Savings |  | Total a without 2-GAE | Eliminated a | 2-GAE performance |
| 1 | 40 | 72 | 4,4 | 0,9793 | Optimal | 42,11% |  | 2,48E+05 | 1,85E+04 | 7% |
| 2 |  | 68 | 4,3 | 1,7401 | ε-Optimal | 44,87% |  | 2,34E+05 | 1,20E+04 | 5% |
| 3 |  | 70 | 4,4 | 1,1606 | Optimal | 40,54% |  | 2,41E+05 | 1,58E+04 | 7% |
| 4 |  | 81 | 5,4 | 1,6889* | Optimal | 28,95% |  | 2,79E+05 | 2,13E+04 | 8% |
| 5 |  | 83 | 5,3 | 1,6879 | ε-Optimal | 41,76% |  | 2,86E+05 | 1,69E+04 | 6% |
| 6 |  | 70 | 4,5 | 1,9593 | Optimal | 44,44% |  | 2,41E+05 | 1,37E+04 | 6% |
| 7 |  | 179 | 10,4 | 1,1434 | Optimal | 24,64% |  | 6,16E+05 | 8,20E+04 | 13% |
| 8 |  | 72 | 4,5 | 2,7044 | Optimal | 32,84% |  | 2,48E+05 | 2,19E+04 | 9% |
| 9 |  | 79 | 5,1 | 1,3649* | ε-Optimal | 44,57% |  | 2,72E+05 | 1,53E+04 | 6% |
| 10 |  | 88 | 5,6 | 1,3638 | Optimal | 44,00% |  | 3,03E+05 | 1,71E+04 | 6% |
| 1 | 60 | 162 | 20 | 0,6162 | ε-Optimal | 53,49% |  | 1,23E+06 | 2,06E+05 | 17% |
| 2 |  | 161 | 20 | 1,0938 | ε-Optimal | 52,38% |  | 1,22E+06 | 2,01E+05 | 17% |
| 3 |  | 163 | 20 | 0,7322 | Optimal | 50,00% |  | 1,23E+06 | 1,88E+05 | 15% |
| 4 |  | 172 | 21 | 1,071 | Optimal | 46,15% |  | 1,30E+06 | 2,21E+05 | 17% |
| 5 |  | 186 | 23 | 1,0713 | ε-Optimal | 51,06% |  | 1,41E+06 | 2,37E+05 | 17% |
| 6 |  | 167 | 21 | 1,2329 | Optimal | 51,16% |  | 1,26E+06 | 1,80E+05 | 14% |
| 7 |  | 223 | 26 | 0,7523 | Optimal | 18,75% |  | 1,69E+06 | 3,59E+05 | 21% |
| 8 |  | 162 | 20 | 1,722 | Optimal | 42,86% |  | 1,23E+06 | 2,09E+05 | 17% |
| 9 |  | 193 | 24 | 0,8572* | Optimal | 48,94% |  | 1,46E+06 | 2,09E+05 | 14% |
| 10 |  | 216 | 26 | 0,8571 | Optimal | 51,85% |  | 1,63E+06 | 2,34E+05 | 14% |
| 1 | 80 | 308 | 63 | 0,4492 | ε-Optimal | 54,35% |  | 4,09E+06 | 9,62E+05 | 24% |
| 2 |  | 277 | 57 | 0,7966 | Optimal | 60,42% |  | 3,68E+06 | 8,46E+05 | 23% |
| 3 |  | 299 | 63 | 0,534 | Optimal | 52,27% |  | 3,97E+06 | 8,68E+05 | 22% |
| 4 |  | 305 | 63 | 0,7826 | ε-Optimal | 52,27% |  | 4,05E+06 | 9,25E+05 | 23% |
| 5 |  | 347 | 72 | 0,7828 | Optimal | 57,65% |  | 4,61E+06 | 1,05E+06 | 23% |
| 6 |  | 312 | 66 | 0,8982 | Optimal | 53,19% |  | 4,14E+06 | 8,76E+05 | 21% |
| 7 |  | 208 | 44 | 0,559 | ε-Optimal | 26,67% |  | 2,76E+06 | 7,21E+05 | 26% |
| 8 |  | 287 | 59 | 1,2611 | Optimal | 46,85% |  | 3,81E+06 | 8,90E+05 | 23% |
| 9 |  | 357 | 74 | 0,6244 | Optimal | 51,95% |  | 4,74E+06 | 1,00E+06 | 21% |
| 10 |  | 370 | 78 | 0,6244 | Optimal | 56,91% |  | 4,92E+06 | 1,03E+06 | 21% |
| 1 | 100 | 469 | 145 | 0,3531 | ε-Optimal | 60,92% |  | 9,66E+06 | 2,69E+06 | 28% |
| 2 |  | 447 | 138 | 0,6265 | Optimal | 60,12% |  | 9,21E+06 | 2,54E+06 | 28% |
| 3 |  | 492 | 156 | 0,4199 | Optimal | 53,15% |  | 1,01E+07 | 2,63E+06 | 26% |
| 4 |  | 488 | 153 | 0,6161 | Optimal | 51,27% |  | 1,01E+07 | 2,70E+06 | 27% |
| 5 |  | 565 | 177 | 0,6163* | Optimal | 55,86% |  | 1,16E+07 | 3,11E+06 | 27% |
| 6 |  | 500 | 158 | 0,7061 | Optimal | 54,99% |  | 1,03E+07 | 2,66E+06 | 26% |
| 7 |  | 299 | 90 | 0,442 | Optimal | 34,31% |  | 6,16E+06 | 1,82E+06 | 30% |
| 8 |  | 462 | 145 | 0,9937 | ε-Optimal | 49,30% |  | 9,52E+06 | 2,60E+06 | 27% |
| 9 |  | 530 | 167 | 0,4908 | Optimal | 56,40% |  | 1,09E+07 | 2,82E+06 | 26% |
| 10 |  | 581 | 181 | 0,4908 | Optimal | 59,69% |  | 1,20E+07 | 3,07E+06 | 26% |
|  | Mean | 47,60% |  |  | Mean | 18% |

| 3-Grade SELSP |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 11 | SVIA |  |  | MDCVIA |  |  |  |  |
| X | k | tCPU | Gk* | k | tCPU | Gk* | Policy | tCPU Savings |
| 15 | 23 | 76 | 6,8986 | 14 | 61 | 6,8715 | ε-Optimal | 19,74% |
| 20 | 32 | 221 | 4,9056 | 17 | 163 | 4,8819 | ε-Optimal | 26,24% |
| 30 | 53 | 1094 | 2,6861 | 23 | 668 | 2,6747 | ε-Optimal | 38,94% |
| 40 | 73 | 3388 | 1,6946 | 31 | 2063 | 1,6859 | ε-Optimal | 39,11% |
| 50 | 85 | 7491 | 1,2147 | 34 | 4221 | 1,2129 | ε-Optimal | 43,65% |
| 60 | 102 | 14933 | 0,9404 | 42 | 8787 | 0,9389 | ε-Optimal | 41,16% |
|  |  |  |  |  |  |  |  | Mean |
|  |  |  |  |  |  |  |  | 34,81% |

| Case 12 | SVIA |  |  | MDCVIA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X | k | tCPU | Gk* | k | tCPU | Gk* | Policy | tCPU Savings |
| 10 | 14 | 330 | 4,1598 | 9 | 310 | 4,1409 | Optimal | 6,06% |
| 15 | 21 | 2014 | 2,6264 | 12 | 1646 | 2,6236 | Optimal | 18,27% |
| 20 | 27 | 6938 | 1,7552 | 14 | 5202 | 1,7578 | ε-Optimal | 25,02% |
| 25 | 28 | 15581 | 1,3051 | 16 | 12116 | 1,3058 | ε-Optimal | 22,24% |
|  |  |  |  |  |  |  |  | Mean |
|  |  |  |  |  |  |  |  | 17,90% |